━━ Wevac

MODEL IMT050

Ice Maker

User Manual

━━━━

Read this manual carefully before use and keep it for future reference.

Illustrations of the product, accessories etc. in this manual are schematic and for reference only. Due to product updates and upgrades, the actual product may differ slightly from the illustrations; the actual product shall prevail.

Wevac	IMT050 — User Manual

Contents

01  Safety Instructions	3

02  Product Usage Tips	5

03  Product Structure	6

04  Product Features	7

05  Specifications	8

06  Operation Guide	9

07  Troubleshooting	11

08  Maintenance	12

09  Installation, Transport, Storage & Disposal	13

10  Brand & Warranty Information	14

Wevac	CH.01 — SAFETY

01 Safety Instructions

For your safety, please read all instructions in this manual before using this electrical appliance and always follow these safety precautions:

WARNING • This product is for household use only. Do not use for any unintended purpose. • Do not allow children to play with this product or its power cord. Children and persons with limited mobility or cognitive ability should use it under adult supervision. • Small parts may pose a choking hazard. Keep away from children. • Do not use this product in high-humidity or high-temperature environments. Never use in the presence of flammable or explosive gases. • Ensure the power supply voltage matches the rating on the product nameplate. • Disconnect the power after use. • When unplugging, grip the plug itself — do not pull the power cord. • Do not immerse the unit or power cord in water or any other liquid. • The power cord must be plugged directly into a properly rated electrical outlet. • If the plug, power cord, or unit is damaged, stop using it immediately and contact the manufacturer or an authorized service center. Do not attempt self-repair. • Do not let the power cord hang over the edge of a counter — children may pull it, causing tipping or injury. • Never touch the product's electrical components with wet hands or bare feet. • The power supply connected to this product must have a residual-current device (RCD/GFCI). • The outlet must comply with local regulations. Safety regulations require mandatory grounding of this product. • Before cleaning, maintenance, or replacing parts, always disconnect the power supply. • Never insert screwdrivers or other objects into the ventilation openings. • Only authorized technicians may disassemble, inspect, or repair this product. • In case of fire, do not use water — use a CO₂ fire extinguisher. • Do not use mechanical means or any method other than those recommended by the manufacturer to accelerate defrosting. • Do not damage the refrigerant circuit. • Warning: Flammable materials — risk of fire. • Refrigerant waste must be disposed of at an authorized waste processing center. Keep away from open flames. • Do not store aerosol cans or other explosive substances containing flammable propellants inside this product. • Keep all ventilation openings clear. Keep the product away from heat sources or steam-generating appliances.

Wevac	CH.01 — SAFETY

01 Safety Instructions (Cont.)

CAUTION • Keep the product upright during transport. Tilting may cause damage. • After transport, allow the unit to stand for 4 hours before powering on, until the refrigerant stabilises. • Wear gloves when servicing the motor or internal refrigeration system. • Do not block the ventilation openings to ensure proper operation. • Do not place non-food items inside the product chamber. • This product is for indoor use only. • This product is designed to make ice from drinking water. Do not use it for any other purpose.

NOTICE • Product noise level below 50 dB(A). • Climate class: SN. Suitable for ambient temperatures of 10℃~32℃ / 50℉~89.6℉ .

Wevac	CH.02 — USAGE TIPS

02 Product Usage Tips

Placement & First Use

•    Place the product on a sturdy, flat, dry countertop.

•    Ensure at least 120mm (4.7") of ventilation clearance around the unit.

•    Keep away from heat sources (ovens, stovetops, etc.) and direct sunlight.

•    Before first use, run a cleaning cycle (see CH.08  Maintenance).

•    For the first 5  ice-making cycles, ice gradually grows larger as water cools — this is normal.

Water Quality

•    Use drinking water for ice making. The purer the water, the clearer the ice.

•    For best clarity, use drinking water with low mineral content (TDS < 200mg/L).

•    Suitable water sources: pure drinking water, RO reverse-osmosis tap water, deeply filtered tap water, low-TDS mineral water.

•    Do not use  coloured, flavoured, carbonated or supersaturated drinks (tea, juice, coffee, milk, soda etc.); they will contaminate or damage the product.

Usage Notes

•    Remove all packaging materials after unboxing, wipe the interior with a damp cloth, and dry thoroughly.

•    When the ice basket is full, the unit pauses automatically and alerts the user.

•    Only one ice size can be produced at a time, but the size can be changed during operation.

•    Occasional cloudy ice during continuous operation is normal and depends on water quality, water temperature, and ambient temperature.

Wevac	CH.03 — STRUCTURE

03 Product Structure

| No. | Name | No. | Name |
| --- | --- | --- | --- |
| 1 | Lid | 5 | Water Tank Handle |
| 2 | Ice Basket | 6 | Control Panel |
| 3 | Ice Mold | 7 | Drain Plug |
| 4 | Water Tank |  |  |

Wevac	CH.04 — FEATURES

04 Product Features

| No. / Button | Description |
| --- | --- |
| 1 Power Power | Press to turn on; long-press to turn off. |
| 2 Make Ice Make Ice | Switch ice size (S / M / L); the unit auto-resumes ice-making. |
| 3 Clean Clean | Start the automatic cleaning cycle. |
| 4 ICE FULL ICE FULL | Indicator (green)— Ice basket full; ice-making paused. |
| 5 ADD WATER ADD WATER | Indicator (red)— Tank low on water; ice-making paused — refill water. |

Wevac	CH.05 — SPECIFICATIONS

05 Specifications

| Parameter | Specification |
| --- | --- |
| Voltage | 220~240 Vac |
| Frequency | 50 Hz |
| Ice-Making Power | 110 W |
| Defrost Power | 130 W |
| Dimensions (W××D××H) | 235 × 305 × 321 mm |
| Net Weight | 7 kg / 15.4 lb |
| 24hIce output | 6.9~7.4 kg |
| Drainage | Manual Drain |
| Ice S | Φ19 × 24 mm |
| Ice M | Φ21 × 25 mm |
| Ice L | Φ22 × 27 mm |
| Ventilation Clearance | ≥ 120 mm / 4.7" |
| Refrigerant | R600a |
| Water Tank Capacity | MAX 1.0 L / MIN 0.75 L |
| Ice Cycle | S: 9pcs/8min, M: 9pcs/10min, L: 9pcs/11.5min |
| Operating Environment | 5~32℃, ≤85% RH |
| Noise Level | ≤ 50 dB(A) |

Wevac	CH.06 — OPERATION

06 Operation Guide

Preparation Before Use

•    Place the unit on a level, stable surface; nothing should obstruct the rear or right-side vents. The clearance from walls/objects must be ≥ 120 mm (4.7 in) for proper ventilation.

•    On first use, or after a long idle period, run one cleaning cycle. See CH.08  Maintenance for cleaning instructions.

Powering On

•    Plug in the unit; you will hear a “beep” and all buttons light then go dark, indicating power.

•    Then press Power; the button shows white — the unit is on.

Making Bullet Ice

1  Open the lid and remove the ice basket and water tank.

2  Fill the bullet-ice water tank with drinking water up to the water level line.

3  Place the water tank back into the bullet-ice chamber. Press down on the tank by hand to make sure it is fully seated. A loose tank may prevent the unit from operating.

Wevac	CH.06 — OPERATION

06 Operation Guide (Cont.)

4  Place the ice basket back in and close the lid.

5     Press Make Ice to switch the lit indicator above S / M / L  and select the bullet-ice size you want.

6  After the size is selected, the unit starts making bullet ice automatically.

ICE FULL →  ADD WATER →

NOTICE • During ice-making you may press Make Ice to change ice size. • During ice-making you may press Power to stop; the bullet-ice button light goes out and the unit stops. • When the bullet-ice basket is detected full, ICE FULL lights green and ice-making stops. Once ice melts or is removed, the unit auto-resumes. • Note: during ice-making, if ADD WATER lights red, the unit stops — refill the water tank promptly.

Wevac	CH.07 — TROUBLESHOOTING

07 Troubleshooting

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| Unit won't turn on | Power not connected or outlet has no power | Check the power connection and ensure the outlet is working. |
| Won't drain | Drain plug not removed or drain hole blocked | Check that the drain plug at the bottom is removed; check for blockages. |
| Not making ice | Tank low on water (ADD WATER lights red) | Fill the water tank to the water line. |
| Not making ice | Mold only frosts, no ice forming (pump issue) | Fill water to the line, let stand for 30 minutes, then restart. Check through the window if water is being pumped to the tray. |
| Stops mid-cycle | Ice basket full (ICE FULL lights green) | Remove ice or wait for it to melt. The unit will resume automatically. |
| Stops mid-cycle | Tank low on water (ADD WATER lights red) | Refill the water tank promptly. The unit will resume automatically. |
| First 5 ice cycles produce small ice | Water temperature is still high (cool-down phase) | This is normal. Ice size will increase as water temperature drops. |
| Poor ice quality | Insufficient ventilation or high ambient temperature | Ensure clearance ≥120mm on all sides; relocate to a cool, ventilated area. |
| Ice is not clear | Hard water or water temperature / / ambient temperature | Use TDS<200mg/L purified water; occasional cloudiness during continuous ice-making is normal. |

Disclaimer DISCLAIMER The manufacturer, distributors, and brand owners shall not be held liable for any damage or quality issues resulting from operations not in accordance with this manual.

Wevac	CH.08 — MAINTENANCE

08 Maintenance

Auto-Clean (Ice Mold & Tray)

1  Fill the water tank to the water line.

2     With the unit on, press Clean; the button lights white and the unit auto-cleans the ice scoop and mould.

3  When cleaning is complete, the unit stops automatically and the indicator turns off.

Draining

1  Unplug the power cord and place the unit near a sink.

2  Remove the drain plug from the bottom of the unit and let all water drain out.

3  Once drained, replace the drain plug securely.

Daily Maintenance

•    Periodically clean the inside with clean water; recommended before each daily use.

•    Wipe the exterior with a soft, damp cloth. Do not use abrasive or corrosive cleaners.

•    Follow the auto-clean steps to clean the ice mold and tray.

•    Always disconnect power before maintenance and wait for the unit to cool down.

Wevac	CH.09 — INSTALLATION / STORAGE / DISPOSAL

09 Installation, Transport, Storage & Disposal

Packaging & Transport

•    Keep the unit balanced during transport to prevent tipping; tilt angle must not exceed 45°.

•    Always keep the product upright during handling and transport to avoid compressor or refrigerant-system damage.

•    After transport, allow the unit to stand for 4  hours before powering on, until the refrigerant stabilises.

Installation

•    Install the product on a flat, stable, and dry surface. Do not operate it while tilted.

•    Do not drag or force the product during positioning to avoid tipping or component damage.

•    Maintain at least 120mm (4.7") clearance to walls/objects for ventilation and serviceability.

•    Keep the product away from heat sources, steam, and direct sunlight.

Storage & Disposal

•    Before long-term storage, drain all water from the tank and inner cavity, then clean and dry the unit thoroughly.

•    Store the unit in a dry, ventilated environment and avoid heavy loads or moisture exposure.

•    This product contains electronic components and R600a  refrigerant; do not dispose of it with household waste.

•    At end of life, contact an authorized recycling center or retailer and follow local disposal regulations.

WEEE / R600a Do not damage the refrigerant circuit. End-of-life treatment must be handled by a qualified recycling or waste-management service to avoid environmental or safety risks.

Wevac	CH.10 — WARRANTY

10 Brand & Warranty Information

Thank you for purchasing the Wevac  Ice Maker. For any questions, contact us via:
support@wevactech.com | www.wevactech.com

Brand Information

| Item | Information |
| --- | --- |
| Brand | WEVAC TECHNOLOGY CO., LIMITED |
| Address | Room 87-105, Park-In Commercial Centre, 404AChatham Road South, Tsim Sha Tsui, Kowloon, Hong Kong |
| Website | www.wevactech.com |
| Customer Support | support@wevactech.com |

Authorized Manufacturer

| Item | Information |
| --- | --- |
| Manufacturer | Guangzhou Argion Electric Appliance Co., Ltd. Guangzhou Argion Electric Appliance Co., Ltd. |
| Address | No. 1, Qiye Road, Nancun Town, Panyu District, Guangzhou, Guangdong |
| Website | www.argiontechnology.com |

Warranty Information

This product is covered by 2  a 2-year limited warranty. Within 2  2 years of the original purchase date you are eligible for warranty replacement. To qualify you must:

•    Have purchased the product within the past 2  2 years.

•    Have the original order number or the email address used at the time of purchase.

•    If purchased through a dealer or retail partner, provide a copy of the original receipt.

Send us an email to apply for warranty service: support@wevactech.com

Wevac	CH.10 — WARRANTY

10 Brand & Warranty Information (Cont.)

Warranty Card

| Customer Name |  |
| --- | --- |
| Address |  |
| City / State / Postal Code |  |
| Phone Number |  |
| Email Address |  |
| Original Purchase Date |  |
| Product Model |  |
| Serial Number (if applicable) |  |
| Retailer Name |  |